DRAFT OWENS-WEIS Requirements

Grey – Requirements not explicitly in the original proposal

Owens is a Multiphysics code for VAWTs that is efficient on remote highly parallel systems

Preferences

- Truly Open Source (in one mode of operation), a well documented and easy to interpret code

Hard and Fast Requirements

OWENS Sub Modules

- OWENS shall have an aerodynamics module that enables efficient aerostructural analysis for optimization studies

- OWENS aerodynamic module shall use the same wind input files as OpenFAST

- OWENS shall use OpenFAST submodules for wind input and hydrodynamics

- Code shall follow literate coding practices

- Code shall follow coding best practices (https://journals.plos.org/plosbiology/article/file?id=10.1371/journal.pbio.1001745&type=printable)

- Main Matlab code shall be compatible with Matlab’s Coder App for C/C++ translation

- Verification shall be done for all of the module interfaces (inflow-aero, aero-structural, static rotor-platform, platform-hydro, platform-mooring), preferably via a direct code-to-code comparison of the VAWT version of HAWC2 or SIMO-RIFLEX-AC

- Validation shall be done for all of the modules, preferably as a floating aero-hydro-servo-elastic system

| Purpose: | Experimental data | Software code-code verification | Code-code tests; simplified model | Unit tests (within a code) – when writing a new subroutine, etc. |
| --- | --- | --- | --- | --- |
| Validation of OWENS modal | SNL 34m | Unnecessary | Unnecessary |  |
| Verification/validation of aero-elastic coupling | Some SNL test with large deflections? (Dale Berg report of SNL 34m VAWT strain levels) | Needed (**HAWC2, SimoReflex) | Needed |  |
| Verification/validation of hydro-elastic coupling | Possibly using SeaTwirl data (SNL through NDA with SeaTwirl). Using the OC6 dataset (**hydrodyn is well validated – focus on code-code) (floaters are rigid to date from test campaigns, not useful for testing platform elasticity models) | Needed ** point to focus on here (**OpenFAST, HAWC2, SimoReflex) (can use to compare different hydrodynamic solvers as well; OrcaFlex) | Needed (possibly with only a tower simulated to compare with OpenFAST) |  |
| Verification (no data available) of full coupled simulation | None available | Needed (**HAWC2, SimoReflex) | Not possible. |  |

WEIS Coupling

- OWENS tool shall work with WEIS so VAWTs can be simulated and optimized with WEIS

- OWENS shall be interchangeable with OpenFAST in the WEIS tool for time-domain simulations

End Project Deliverables

- OWENS and associated documentation shall be released publicly

- A more optimal integration shall be determined as a result of the project

- Capability of and effort for linearization of the OWENS VAWT analysis shall be determined

The statements from the original collaboration tasks make the framework look like the following, consistent with what we have previously decided:

Collaboration Wishlist:

- Frequent call/in-person meetings of the group to keep unity

- A self-regulating way to coordinate the distribution of work, which is easy for PI to assess progress, change priority, and update upcoming work (i.e. a type of shared Kanban board in gitlab or rally – it looks like MS Teams may be able to do this, but Sandia won’t have the ability until at least Sept)

- A way to quickly communicate small questions/answers based on subject (something like slack, or gitlab issues, or rally – I guess we could just use text messaging too), with ability to share snippets of code/documents. – This could be handled by teams

- A way to simultaneously make changes to code/test cases/documentation – Potentially handled by git and a common repo

- A process for reviewing and accepting content/code changes (preferably with automated testing for code) – Potentially handled by the repo host (i.e. gitlab/hithub), with a content checklist